TCES 330

John Teal

Seth Amico

## Part 1:

Translate the following pseudocode into a sequence of legal instructions; represent machine code in hex.
RF[0A] = D[1B] - D[2A] + D[3C] - D[7E];

D[6A] = RF[0A];

asm:

LDR	1B	 0

LDR	 2A	 1

LDR	 3C	 2

LDR	 7E  3

SUB	 0   1   0

ADD 	 0   2   0

SUB 	 0   3   0

STR	 0A  6A

HLT

%machine code

0010	00011011  0000

0010	00101010  0001

0010	00111100  0010

0010	01111110  0011

0100	0000 0001 0000

0011	0000 0010 0000

0100	0000 0011 0000

0001	1010 0110 1010

0101	0000 0000 0000

#HEX

21B0

22A1

23C2

27E3

4010

3020

4030

1A6A

5000

HALT

## Part 2:

The ALU module was tested by linking it to the DE-2 board according to the description below. The Verilog module was tested for each function, and 256 randomized instructions and data inputs.

### ALU to DE-2 hardware mapping, taken from SV module header

The 16 switches set either aluA (shown on hex7-4) or aluB (shown on hex3-0) registers, based on the state of switch 15.  Switch 15 on is aluA display on, switch 15 off is aluB hex display on. The registers then feed into the inputs of the ALU, which has its operation run by CLOCK_50. The alu then is operated by positive clock edge to output to the 16 red LEDs below the switches. KEY 3-0 (on LEDG 3-0) determine the ALUop.