Course Project

Programmable Processer

TCES330 Digital System Design

Spring 2026

Authors:

Student A: Seth Amico

Student B: John R Teal Jr.

Submission Date: June 10, 2026

Table of Contents:

1: Project requirements	    _________________________________________ 4

1.1 Controller

1.2 Datapath

1.3 Processor

1.4 Quartus Project

2: Design Details                        _________________________________________ page number

2.1 Controller

2.2 Datapath

2.3 Processor

2.4 Project

3: Test Procedures and Results   ________________________________________ page number

3.1 FSM_tb.sv

3.2 Controller_tb.sv

3.3 Datapath_tb.sv

3.4 testProcessor.sv

4: DE Board Test Results	    _________________________________________ page number

5: Conclusion			    _________________________________________ page number

6: References

Appendix

The purpose of this course project was to design, simulate, integrate, and test a small programmable processor using SystemVerilog. The processor was built from several smaller modules, including a finite state machine, program counter, instruction register, ROM, RAM, register file, ALU, controller, datapath, and top-level board interface. By completing the project in stages, the design process reinforced how individual digital system components work together to form a functioning processor. The final processor was verified through ModelSim simulations and then tested on the DE10 board using manual clock stepping, synchronous reset, switch-controlled debug output, and seven-segment displays.

Our team members for this project were Seth Amico and John Teal. We managed the workload by dividing the processor into smaller design and verification tasks, then combining those pieces through several stages of integration. Seth contributed to the early project structure and module development, including the initial datapath work, RAM and ROM setup, control unit development, program counter work, and early integration between the controller-related modules. John contributed to module refinement, interface corrections, FSM updates, datapath debugging, testbench creation, simulation setup, processor integration, top-level Project.sv board integration, Decoder and KeyFilter support, Quartus cleanup, timing constraints, and DE10 board testing. Both team members contributed to debugging and integration as the design moved from individual modules to the final processor and board-level demonstration.

1.Requirements

Briefly describe the project requirements in general. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. See below.

Add your picture here

Figure x. Brief describe the picture you insert here, e.g., Processor Structure, etc

  - Requirements for Controller

Briefly describe design requirements for the module Controller. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references.

16

8

8

PC

State Machine

IR

Instruction memory (ROM)

16

addr

w_en

select

R/W controls

function select

ALU flags

Figure 1: Datapath module flowchart depicting Data Memory, source mux, Register file, and ALU

Data Memory RAM

Register File

Arithmetic Logic Unit

addr

w_en

select

R/W controls

function select

16-bit 2-1 mux

16

16

16

16

16

16

ALU flags

Requirements for Datapath

Briefly describe the design requirements for the module Datapath. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references

The Datapath module is one of the two critical sections of the processor. It houses the Arithmetic Logic Unit (ALU), the Register File (RF) and the Data Memory (RAM). The Data Memory acts as the Random Access Memory (RAM) of the processor, holding 256 16-bit addresses. It can be read or written to synchronously, reading from the RF or writing values back to the RF. The Register File houses 16 16-bit registers, allowing 2 simultaneous synchronous read operations or one synchronous write operation. Both of the RF’s output ports are wired as inputs to the ALU, with output A being connected to the data input of the RAM additionally. The RF takes data in from the the ALU’s output port or the RAM’s output port. The ALU submodule reads data from two 16-bit input register buses and asynchronously performs one of 8 operations based on the state of the 3-bit function select bus. Additionally, it sets three output flags based on the result of the oprtation. These three modules are augmented by a 16-bit two-to-one mux, which selects the input of the RF from the ALU output or the RAM output.

The Datapath module is interfaced with by the following wires. The RAM submodule exposes an 8-bit data address line, a write enable line, and a global clock input. The control mux exposes its select line, with a logic high selecting the RAM as a source. The RF submodule exposes 3 3-bit address select lines: one read and two write. This is joined by a write enable line and clock input. The ALU submodule exposes a 3-bit function select line, as well as 3 individual flags: Z, N, and V.

  - Requirements for Processor

Briefly describe the design requirements for the module Processor. Use pictures/figures is always a good idea. When using pictures/figures, please add captions for references.

  - Requirements for Project

Briefly describe the design requirements for the top-level module Project (by adding other modules like Buttonsync, Keyfilter, Decoder, Mux_nW_8_to_1 to the processor). Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references.

2. Design

Describe in detail the project modules’ design procedure. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references.

2.1 Controller.sv

Describe in detail the design of the Controller module. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. If you think certain segments of the SystemVerilog codes can reflect your design idea, you can add them here for discussion too (and don’t forget to add captions).

2.2 Datapath.sv

Describe in detail the design of the Datapath module. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. If you think certain segments of the SystemVerilog codes can reflect your design idea, you can add them here for discussion too (and don’t forget to add captions).

2.3 Processor.sv

Describe in detail the design of the Processor module. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. If you think certain segments of the SystemVerilog codes can reflect your design idea, you can add them here for discussion too (and don’t forget to add captions).

2.4 Project.sv

Describe in detail the design of the top level Project module. Discuss the usage of submodules including Button Synchronizer, Key Filter, etc. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. If you think certain segments of the SystemVerilog codes can reflect your design idea, you can add them here for discussion too (and don’t forget to add captions).

3. Test Procedures and Results

Describe in detail the testbench design of each module. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. If you think certain segments of the SystemVerilog codes can reflect your design idea, you can add them here for discussion too (and don’t forget to add captions).

Also please include the simulation results by running each testbench. Comment properly on whether the testing results meet the design requirements presented in Section 1: Requirements.

3.1 FSM_tb

Describe in detail the design of FSM testbench. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. If you think certain segments of the SystemVerilog codes can reflect your design idea, you can add them here for discussion too (and don’t forget to add captions).

3.2 Controller_tb

Describe in detail the design of the Controller testbench. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. If you think certain segments of the SystemVerilog codes can reflect your design idea, you can add them here for discussion too (and don’t forget to add captions).

3.3 Datapath_tb

Describe in detail the design of the Datapath testbench. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references. If you think certain segments of the SystemVerilog codes can reflect your design idea, you can add them here for discussion too (and don’t forget to add captions).

3.4 testProcessor

The Processor testbench, testProcessor.sv will be provided. Show your understanding of the test bench. If you have ever tried anything different from the given version, properly talk about it and explain why your version is a reasonable (or preferable) alternative.

4. DE Board Test Results

Provide your observations through project testing on DE2/DE10 board. If you or your group ever encountered some warning messages, or error messages during Quartus compilation, how your group solved the problem and removed those warning/error messages. Using pictures/figures is always a good idea. When using pictures/figures, please add captions for references.

Each group should record a narrated video demonstrating the project in operation and upload the video to the Google Drive folder. Please name your video in the format of Lastname1_Lastname2_Lastname3.

5. Conclusion

Provide a summary of your learning from the course project, learning of course subjects, as well as learning from the teamwork experience.

6. References

List documents/materials/webpages you and/or your group referred to when you work on this project and when you write this report.

Appendix

If you and/or your group has some extra information/work to share, please put it in this section. As an example, if you tried the extra credits part, you can have description and discussion on those contents here.